Speed Limit Note:

INSTALL:

1  -  NPAT BOTH SIDES THROUGHOUT ENTIRE PROJECT WHERE

CURB & GUTTER EXIST.

2  -  R2-1(45) 4 PER SIDE PER MILE.

3  -  MAINTAIN EXISTING SIGNS UNLESS OTHERWISE NOTED.

Bike Note:

NOTE:

1 -  INSTALL NPAT BOTH SIDES THROUGHOUT ENTIRE PROJECT WHERE

CURB EXISTS UNLESS OTHERWISE NOTED.

2 -  INSTALL APPROX. 2 R3-17 PER MILE DEPENDING

ON ENGINEER'S JUDGEMENT.

3 -  PAINT APPROX. 4 BIKE SYMBOLS IN BIKE LANE PER

MILE DEPENDING ON ENGINEER'S JUDGEMENT.

4 -  END BIKE LANE 50 FT FROM CURB RETURN OF UNSIGNALIZED

INTERSECTIONS ON APPROACH UNLESS OTHERWISE NOTED.

5 -  END BIKE LANE 200 FT FROM RIGHT TURN LANE ON APPROACH

UNLESS OTHERWISE NOTED.

6 -  END BIKE LANE 200 FT FROM SIGNALIZED INTERSECTIONS

ON APPROACH UNLESS OTHERWISE NOTED.

7 -  MAINTAIN EXISTING SIGNS UNLESS OTHERWISE NOTED.