LIST OF NOTES TO BE INCLUDED IN TRAFFIC SIGNAL PLANS

1.	CITY OF PHOENIX ORDINANCE G-4396

Per City of Phoenix Ordinance G-4396, these plans are for official use only and may not be shared with others except as required to fulfill the obligations of your contract with the City of Phoenix.

2.	POLE/SIDEWALK CLEARENCE NOTE

A Minimum 3 Feet Of Clearance Is Required Between All Traffic Signal Poles And Sidewalk Ramps.

- CONTRACTOR NOTE FOR CITY OF PHOENIX POLE

(Show this note only when contractor is providing special poles)

  - The contractor shall provide and install ___ City of Phoenix type _________ pole(s) with anchor cage, nuts & bolts, luminaire riser, 8’ Luminaire mast arm and ______ ft mast arms as per C.O.P. St. Trans> Dept. drawing #7690.

  - Signal Poles, Mast Arms, mast Arm Extensions, luminaire extensions, and 8’ luminaire mast arm shall be power coated. Contact the Traffic Signal Construction Supervisor at (602) 262-6733 to confirm color.

  - Separate pull wires shall be installed from each tendon on the Signal Mast Arm and the Luminaire Mast Arm to the hand hole at the base of the Traffic Signal Pole.

4.	WORK TO BE PERFORMED BY CONTRACTOR:

Contractor to provide all underground work including but not limited to:

- All trenching

- All foundations

- All Junction boxes w/ 18” of sump

- All Conduit runs with #10 bond wire installed

- Provide and install the power service pedestal and foundation

- Traffic Signal equipment as indicated in note 3.1

5.	WORK TO BE PERFORMED BY CITY OF PHOENIX CREWS:

City Of Phoenix Traffic Signal Crews Will Provide and Install All Aboveground traffic Signal Equipment including:

- All Poles except as indicated in note 3.1

- All Signal Heads

- All Pedestrian Heads

- Pedestrian Push Buttons

- Controller

- All Luminaries

- All Illuminated Street Signs

- All Signal Wiring

- All Loop Detectors

- WARRANTED SIGNALS:

(Show this note only on warranted signals)

All work to performed by City Of Phoenix Traffic Signal Crews.